1. Install OpenOCD for ESP32

- Reference:

  - https://docs.espressif.com/projects/esp-idf/en/v3.3/api-guides/jtag-debugging/setup-openocd-windows.html

- Download OpenOCD for Windows

  - OpenOCD for Windows is available for download from Github: https://github.com/espressif/openocd-esp32/releases

- Extract OpenOCD package. Add OpenOCD to path environment.

- Check version of OpenOCD

  - openocd.exe –version

2. OpenOCD for ESP32 use FT2232 minimodule

- Reference

  - https://mcuoneclipse.com/2019/10/20/jtag-debugging-the-esp32-with-ft2232-and-openocd/

  - https://docs.espressif.com/projects/esp-idf/en/v3.3/api-guides/jtag-debugging/index.html#jtag-debugging-configuring-esp32-target

3. Debugging ESP32 by JTAG with OpenOCD + GDB on Visual Studio Code for Arduino

- JTAG Connection

  - IO12: JTAG TDI

  - IO13: JTAG TCK

  - IO14: JTAG TMS

  - IO15: JTAG TDO

  - RST/EN: JTAG Reset

  - 3.3V: VTref

- Install Arduino Extension on Visual Studio Code

- Install ESP32 package

- Create Arduino Blink project from example

  - F1 => Typing “Arduino: Examples”

  - Config board type for Blink project

  - Config output directory for build output data

  - Build Blink project

  - Create OpenOCD Server task

    - F1 => Typing “Tasks: Configure”

    - Select “Tasks: Configure Task”

    - Select “Create task.json file from template”

    - Select “MSBuild Executes the build target”

    - Edit task

    - task.json reference content

- { - // See https://go.microsoft.com/fwlink/?LinkId=733558 - // for the documentation about the tasks.json format - "version": "2.0.0", - "tasks": [ - { - "label": "OpenOCD Server", - "type": "shell", - "isBackground": true, - "options": { - "cwd": "${workspaceFolder}/build" - }, - "command": "clear ; openocd.exe -s share/openocd/scripts -f interface/jlink.cfg -f board/esp-wroom-32.cfg", - "args": [ - ], - "group": "build", - "presentation": { - // Reveal the output only if unrecognized errors occur. - "reveal": "silent" - }, - // Use the standard MS compiler pattern to detect errors, warnings and infos - "problemMatcher": "$msCompile" - }, - { - "label": "Flash Image", - "type": "shell", - "options": { - "cwd": "${workspaceFolder}/build" - }, - "command":[ - "clear ; cd ${workspaceFolder}/build ; openocd.exe -s share/openocd/scripts -f interface/jlink.cfg -f board/esp-wroom-32.cfg -c 'program_esp ./Blink.ino.bin 0x10000 verify reset exit'" - ], - "args": [ - ], - "group": "build", - "presentation": { - // Reveal the output only if unrecognized errors occur. - // "reveal": "silent", - "close": true// Close terminal when task command execute complete - }, - // Use the standard MS compiler pattern to detect errors, warnings and infos - "problemMatcher": "$msCompile" - }, - { - "label": "Terminate Task", - "type": "shell", - "options": { - "cwd": "${workspaceFolder}/build" - }, - "command":[ - "${command:workbench.action.tasks.terminate}", - "${command:workbench.action.acceptSelectedQuickOpenItem}", - ], - "args": [ - ], - "group": "build", - "presentation": { - // Reveal the output only if unrecognized errors occur. - // "reveal": "silent", - "close": true - }, - // Use the standard MS compiler pattern to detect errors, warnings and infos - "problemMatcher": "$msCompile" - }, - { - "label": "Exit OpenOCD", - "type": "shell", - "options": { - "cwd": "${workspaceFolder}/build" - }, - "command": "echo ${input:terminate}", - "args": [ - ], - "group": "build", - "presentation": { - // Reveal the output only if unrecognized errors occur. - // "reveal": "silent", - "close": true - }, - // Use the standard MS compiler pattern to detect errors, warnings and infos - "problemMatcher": "$msCompile" - }, - ], - "inputs": [ - { - "id": "terminate", - "type": "command", - "command": "workbench.action.tasks.terminate", - "args": "OpenOCD Server" - } - ] - }

    - Edit jlink.cfg file

      - Add adapter speed parameter

    - Check command of task

    - Check task by run task from Visual Studio Code

      - F1 => Typing “Tasks: Run”

      - Select “Tasks: Run Task” => Select “OpenOCD Server” task

  - Create “gdb Launch”

    - Press “Ctrl + Shift + D”

    - Select “Show all automatic debug configurations”

    - Select “Add configuration…”

    - Select “C++ (GDB/LLDB)”

    - Select “Add Configuration”

    - Select “C/C++: (gdb) Launch”

    - Edit config

    - launch.json reference content

- { - // Use IntelliSense to learn about possible attributes. - // Hover to view descriptions of existing attributes. - // For more information, visit: https://go.microsoft.com/fwlink/?linkid=830387 - "version": "0.2.0", - "configurations": [ - { - "preLaunchTask": "OpenOCD Server",// Start OpenOCD Server first - "name": "(gdb) Launch", - "type": "cppdbg", - "request": "launch", - "program": "${workspaceFolder}/build/Blink.ino.elf", - "args": [], - "stopAtEntry": false, - "cwd": "${workspaceFolder}/build", - "environment": [], - "externalConsole": false, - "MIMode": "gdb", - "miDebuggerPath": "C:/Users/137151/AppData/Local/Arduino15/packages/esp32/tools/xtensa-esp32-elf-gcc/gcc8_4_0-esp-2021r2-patch3/bin/xtensa-esp32-elf-gdb.exe", - // Connect to OpenOCD server - "setupCommands": [ - { - "description": "Enable pretty-printing for gdb", - "text": "-enable-pretty-printing", - "ignoreFailures": true - }, - { - "description": "Command 1", - "text": "target remote localhost:3333", - "ignoreFailures": true - }, - ], - // Seperate with setupCommands to fix issue "Info : dropped 'gdb' connection" - "postRemoteConnectCommands": [ - { - "description": "Respect Hardware Limitations (as prescribed by Espressif)", - "text": "set remote hardware-watchpoint-limit 2", - "ignoreFailures": false - }, - { - "description": "Hard Reset and Immediately Halt", - "text": "monitor reset halt", - "ignoreFailures": true - }, - { - "description": "Flush Internal Register Cache", - "text": "flushregs", - "ignoreFailures": false - }, - // { - // "description": "Set Temporary Hardware Assisted Breakpoint at `app_main`", - // "text": "thbreak setup", - // "ignoreFailures": false - // }, - // { - // "description": "Shutdown GDB Server on GDB Detach", - // "text": "monitor [target current] configure -event gdb-detach { shutdown }", - // "ignoreFailures": true - // }, - { - "description": "Reset MCU when stop debug session", - "text": "monitor [target current] configure -event gdb-detach { reset }", - "ignoreFailures": true - }, - ], - "launchCompleteCommand":"None",// To fix issues [ERROR: Unable to start debugging. Unexpected GDB output from command "-exec-run". Don't know how to run. Try "help target".] - // "internalConsoleOptions": "openOnSessionStart", - // "logging": { "engineLogging": true },// Enable log of GDB commands - "postDebugTask": "Exit OpenOCD" - } - ] - }

  - Start Debug

    - Run => Start Debugging

    - If error: The task ‘OpenOCD Server’ cannot be tracked. Select Debug Anyway.

    - If fail try to 3 times it will worked.

  - Log output from GDB server

- Debug flow:

  - Build project

  - Flash firmware to ESP32

    - F1 => Tasks: Run Task => Flash Image

  - Start debug

    - F5

  - Stop debug. Don’t restart.

4.